Assignment

Computer Networks

Name: Aamir Hussain

CMS: 023-22-0080

# Guided and Unguided Media , Frequencies of Different Channels

- Guided Media:

# Guided media, also known as wired or bounded media, are physical pathways that transmit data signals along a defined path. Examples include twisted pair cables, coaxial cables, and fiber optic cables. These cables provide a conduit for signals, guiding them from one device to another.

Characteristics:

- Signal propagation is confined within physical boundaries.

- Less susceptible to environmental interference.

- Offers higher bandwidth and better security.

Types of Guided Media:

- Twisted Pair Cable: Consists of pairs of insulated copper wires twisted together.

  - Used in: Telephone networks, LANs (Ethernet).

  - Frequency range: Up to 100 MHz (for Cat5e/Cat6 cables).

  - Advantages: Cheap, easy to install.

  - Disadvantages: Susceptible to electromagnetic interference (EMI).

- Coaxial Cable: Comprises a central core conductor, insulating layer, metal shield, and outer cover.

  - Used in: Cable television, broadband internet.

  - Frequency range: Up to several GHz (3–10 GHz typically).

  - Advantages: Better shielding than twisted pair.

  - Disadvantages: More expensive and rigid.

- Fiber Optic Cable: Uses light to transmit data through glass or plastic fibers.

  - Used in: High-speed internet, long-distance communication.

  - Frequency range: Operates in the optical range (typically 186–230 THz).

  - Advantages: Immune to EMI, extremely high speed and bandwidth.

  - Disadvantages: Expensive and fragile, complex installation.

### 2. Unguided Media (Wireless Communication)

Definition: Unguided media transmits data through the air or space without the use of physical conductors.

Characteristics:

- Signal radiates in all directions.

- More prone to noise, interference, and weather effects.

- Enables mobility and broad coverage.

Types of Unguided Media:

- Radio Waves: Electromagnetic waves with long wavelengths and low frequency.

  - Used in: AM/FM radio, mobile phones, Wi-Fi.

  - Frequency range: 3 kHz to 1 GHz.

  - Advantages: Penetrates walls, good for long-distance and outdoor communication.

  - Disadvantages: Limited bandwidth, susceptible to interference.

- Microwaves: High-frequency radio waves that travel in straight lines.

  - Used in: Satellite links, radar, cellular networks.

  - Frequency range: 1 GHz to 300 GHz.

  - Advantages: Suitable for point-to-point communication.

  - Disadvantages: Needs line-of-sight, can be affected by rain (rain fade).

- Infrared and Satellite Communication:

  - Infrared: Short-range, used in remote controls and wireless peripherals.

  - Satellite: Utilizes satellites to relay signals.

  - Frequency range: 2 GHz to 40 GHz (for satellite uplink/downlink).

  - Advantages: Wide coverage, useful in remote areas.

  - Disadvantages: Expensive, delays due to distance.

Summary Table:

| Feature | Guided Media | Unguided Media |
| --- | --- | --- |
| Transmission Path | Through cables or physical materials | Through air or vacuum |
| Examples | Twisted pair, coaxial, fiber optics | Radio waves, microwaves, satellite signals |
| Security | High | Lower |
| Interference | Low | High |
| Installation Cost | Moderate to High (depends on type) | Lower (for local coverage) |
| Mobility | Limited | High |

The End!